# RFID Trigger Configuration & Event Subscription

Release: v1.0.0

## 1. Overview

The RFIDHandler class manages the Zebra RFID reader’s hardware trigger, which can operate in two modes: - RFID Mode: Trigger starts/stops RFID tag inventory. - Barcode (Sled Scan) Mode: Trigger activates the barcode scanner.

The application dynamically reconfigures the hardware key layout and manages event subscriptions to support these modes. This document describes the design and logic for: - setTriggerEnabled (hardware configuration) - subscribeRfidHardwareTriggerEvents (software event masking)

## 2. Architecture & Synchronization

### The Challenge

Reconfiguring the hardware trigger (setKeylayoutType) requires sending commands to the reader. If the reader is busy (e.g., inventory in progress) or if multiple threads attempt configuration simultaneously, errors or undefined states may occur.

### Synchronization Strategy

To ensure thread safety and hardware stability: - resourceLock (ReentrantLock): Only one configuration operation at a time. - bRfidBusy (volatile boolean): Blocks configuration commands while the reader is busy. - waitForReaderIdle(): Helper to avoid indefinite waits during restore operations.

## 3. Key Design Elements

### 3.1 Busy Guard: bRfidBusy

- Purpose: Indicates if the reader is busy (e.g., inventory running).

- Set to true: On INVENTORY_START_EVENT.

- Set to false: On INVENTORY_STOP_EVENT.

- Usage: Blocks configuration changes while busy; operations are rejected and UI is notified to retry.

- waitForReaderIdle(): Polls bRfidBusy with a timeout before configuration changes.

### 3.2 Mode Switch Guard: bSwitchFromRfidToBarcode

- Purpose: Prevents RFID trigger events from being processed during a switch to barcode mode.

- Set to true: When inventory stops in test mode, to ignore subsequent RFID trigger events.

- Set to false: When switching back to RFID mode.

- Usage: If true, RFID trigger events are ignored.

Sequence Example: 1. Inventory stops in test mode → bSwitchFromRfidToBarcode = true 2. Trigger event occurs → RFID handler ignores it 3. Barcode mode is enabled 4. Switching back to RFID → flag reset

### 3.3 Hardware Trigger Configuration: setTriggerEnabled(boolean isRfidEnabled)

- Acquire Lock: Prevents concurrent configuration.

- Check Busy Guard: Rejects if busy, stops inventory, notifies UI.

- Determine Mode: true for RFID, false for Barcode.

- Apply Configuration: Calls reader.Config.setKeylayoutType.

- Align Event Subscription: Calls subscribeRfidHardwareTriggerEvents(isRfidEnabled) on success.

- Release Lock: Always releases in a finally block.

### 3.4 Event Subscription: subscribeRfidHardwareTriggerEvents(boolean enable)

- Purpose: Controls whether the app receives trigger events from the SDK.

- Enable (true): App receives trigger callbacks (needed for RFID mode).

- Disable (false): App ignores trigger events (used in Barcode mode).

- Integration:

  - Switching to Barcode: Unsubscribe events, then reconfigure hardware.

  - Switching to RFID: Reconfigure hardware, then subscribe events.

## 4. Integration Workflow

Switching to Barcode Mode: 1. Unsubscribe events: subscribeRfidHardwareTriggerEvents(false) 2. Reconfigure hardware: setTriggerEnabled(false)

Switching to RFID Mode: 1. Reconfigure hardware: setTriggerEnabled(true) 2. On success, events are automatically subscribed.

## 5. Safety Mechanisms

| Mechanism | Purpose |
| --- | --- |
| ReentrantLock | Prevents concurrent configuration operations |
| waitForReaderIdle | Avoids sending commands while the reader is active |
| Timeout (~3s) | Prevents hangs during restore operations |
| Try-Catch-Finally | Ensures locks are always released, even on exceptions |

## 6. Deadlock Avoidance

- Never block indefinitely while holding resourceLock.

- waitForReaderIdle() uses bounded retries and fails fast.

- bRfidBusy is volatile and updated directly by status events.

- Always release locks in all code paths.

- Switch order: Unsubscribe events before hardware switch (RFID→Barcode); enable events after hardware switch (Barcode→RFID).

## 7. Code Reference

```java public boolean setTriggerEnabled(boolean isRfidEnabled) { resourceLock.lock(); try { if (reader == null || !reader.isConnected() || context == null) return false; if (bRfidBusy) { stopInventory(); uiHandler.post(() -> { if (context != null) context.showSnackbar(BUSY_RETRY_MESSAGE, false); }); return false; } ENUM_NEW_KEYLAYOUT_TYPE mode = isRfidEnabled ? ENUM_NEW_KEYLAYOUT_TYPE.RFID : ENUM_NEW_KEYLAYOUT_TYPE.SLED_SCAN; try { RFIDResults result = reader.Config.setKeylayoutType(mode, mode); if (result == RFIDResults.RFID_API_SUCCESS) { subscribeRfidHardwareTriggerEvents(isRfidEnabled); if(isRfidEnabled) bSwitchFromRfidToBarcode = false; return true; } } catch (InvalidUsageException | OperationFailureException e) { stopInventory(); uiHandler.post(() -> { if (context != null) context.showSnackbar(BUSY_RETRY_MESSAGE, false); }); } return false; } finally { resourceLock.unlock(); } }