# Trigger Sync & Deadlock-Safe Switching

Release: v1.0.0

## Purpose

Safely switch the RFD40 trigger between RFID inventory and barcode scanning modes, avoiding SDK errors and deadlocks.

## Concurrency Model

- Single worker thread: Serializes all operations

- Lock (resourceLock): Wraps all trigger config methods

- Busy flag (bRfidBusy): Tracks RFID operation state

## Busy Guard Logic

- bRfidBusy = true on inventory start, false on stop

- All config changes check bRfidBusy before proceeding

- waitForReaderIdle() polls with timeout to ensure idle

## Safe Switch Sequence

- RFID → Barcode: Unsubscribe events, then switch hardware

- Barcode → RFID: Switch hardware, then subscribe events

## Failure Handling

- On timeout or SDK error, return false and release lock

- Never block indefinitely or leave lock held

## Verification

- Press trigger repeatedly during mode switch: app must not freeze

- Confirm trigger events are ignored in barcode mode, resume in RFID mode

## See Also

- design.md for architecture

- README.md for usage

- RELEASE_NOTES.md for release info